To display hidden notes to specifier, from pull down menu “Tools’/”Options”/”View”/”Hidden Text”

Action Bullet Resistant, Inc. – BR 250 Door

SECTION 083453 – SECURITY DOORS AND FRAMES

PART 1 – GENERAL

  - SECTION INCLUDES

- Aluminum bullet and blast resistant interior and exterior security doors.

  - RELATED DOCUMENTS

- Drawings, and general provisions of the Contract.

  - RELATED SECTIONS

- Section 088856 – Ballistics-Resistant Glazing

- Section 087100 – Door Hardware

- Section 013300 – Submittal Procedures

  - PERFORMANCE REQUIRMENTS

- Ballistics-Resistance Performance Requirements: Provide security door assemblies identical to those tested for compliance with requirements indicated, and as follows.

  - Security door assemblies will meet or exceed the requirements of

UL 752 Level (1,2,3,4,5,6,7,8)

- Testing is conducted by H.P. White Laboratory, or an approved equivalent independent testing laboratory.

- Proof of certification is available upon request.

- Testing conforms to Underwriters Laboratory UL 752 Ballistic Standards.

- Blast Resistant Performance and Design Requirements: Provide security door assemblies and the anchorage analysis that comply with requirements indicated and as follows.

  - Security door assemblies will be calculated to meet:

Note to specifier: Insert blast level GSA Level C of 4 psi or DOD UFC criteria of a standoff distance of 33ft. or 82ft. If a greater blast criteria is desired contact a sales representative at Action Bullet Resistant.

  - Security door assemblies will comply with design build blast drawings and calculations.

  - A detailed engineered blast calculations by a qualified blast engineer to substantiate that the system design and anchorage meets or exceeds the minimum performance required.

  - SUBMITTALS

- Design Calculations: Submit design analysis and calculations verifying conformance to specified blast criteria.

- Shop Drawings: Provide elevations indicating rough openings requirements and details for field installed assemblies.

- Qualification Data: Submit qualifications verifying years of experience; include list of completed jobs having similar scope of work, identified by name, location, date contact names and phone numbers.

- Product Data: Provide manufacture’s data sheets on all products used.

- Selection Samples: Provide two product samples for each finish specified.

- Verification Samples: Provide sample for product specified, minimum size 6 inches (150 mm) square, and representing actual product.

  - QUALITY ASSURANCE

- Provide test reports from a nationally recognized testing laboratory certifying the UL rating on product.

- Drawings and specifications are based on Action Bullet Resistant’s Model BR 250 Door.

- Installer Qualifications: An experienced installer who has completed similar design, material and extent to this project.

- Manufacture’s facility will be made available during production for inspection of doors by customer’s representative to ensure compliance of products with drawings and specifications.

  - DELIVERY, STORAGE AND HANDLING

- Product will be securely crated to prevent damage during shipping. Each crate will be identified by door number or elevation. Packing list will identify all contents of crate.

- Verify contents of crate within five days of receipt. If crates are to be stored for long periods due to construction delays, place in a dry warehouse with moderate temperatures.

- When handling crates use forklift or pallet jack.

  - PROJECT CONDITIONS

- Field Measurements: Verify security door openings by field measurements before fabrication and indicate measurements on Shop Drawings.

- Established Dimensions: Where field measurements cannot be made without delaying the work, establish openings and proceed with fabrication without field measurements using held dimensions. Coordinate wall construction to ensure that actual openings dimensions correspond with held dimensions.

  - WARRANTY

- Contractor warrants installation for a period of one year and assumes full responsibility for installation of the system, which includes the door, glass, anchorage, setting, sealing, flashing, etc, as it relates to air, water and structural adequacy as required in the shop drawings and specifications.

- Manufacture warrants products for period of one year from the date of customer receipt.

PART 2 – PRODUCTS

  - MANUFACTURE

- Acceptable Manufacturer:

Action Bullet Resistant, Inc., located at 235 Union Blvd. West Islip, NY 11795:

Toll Free Tel. 800-962-8088; Email info@actionbullet.com: Web www.actionbullet.com

  - PRODUCT TYPE

- Product available in full vision, half panel or full panel. Single swing or pair of doors with astragal, fixed or removable mullion.

  - MATERIAL

- Aluminum Extrusions: Aluminum alloy and temper – 6005-T5. Ultimate tensile strength of 37.7 ksi, and shear strength of 29.7 ksi.

- Steel Blast Reinforcement: (Steel straps and anchors)

- Steel plates, shapes and bars 	ASTM A-36

- Seamless steel structural tubing 	ASTM A-500.

- Silicone: Structural silicone GE SCS 1000 complies with ASTM-C-920-05.

- Glass and Glazing: See Section 088856.

- Setting blocks – Neoprene 80-85 durometer ASTM-D-2000

- Hardware: For custom hardware, contact Action Bullet Resistant’s sales representative for hardware compatibility. Standard door hardware:

  - Select SL21HD full surface hinge.

  - Adams Rite 1850 lock with standard keyed cylinder and thumb turn. Class 1 surface mounted closer.

  - Rockwood 1” diameter 10” center offset wire push/pull set.

  - Threshold ½” x 4” aluminum mill finish.

  - FABRICATION

- Door is a heavy-duty aluminum medium style. Door thickness 2 ¼” with a 3/8” minimum wall thickness, vertical door stile 4” wide, door frame profile 2 ½” x 4 ½” with a 3/16” minimum wall thickness. Glazing pocket to be 1” deep for proper bite.

- Horizontal top door rail to be nominal 4”, additional rails can be added upon request.

Note to specifier: Modify door rail dimension to accommodate additional rails.

- Horizontal bottom rail is 5 ½” wide with a removable stop for glazing in the field, additional rails can be added upon request.

Note to specifier: Modify door rail dimension to accommodate additional rails.

- Doors to have tight hairline joints where rails are fitted against stiles.

- Door stops snap in as an integral part of door frame.

- Door accommodates a maximum glass thickness of 1 3/8”.

- Doors and frames are fully fabricated and shipped to customer for field installation.

- Fasteners where exposed will be stainless or other non-corrosive material.

- Doors are glazed from the inside by utilizing the removable stop and wet sealed.

  - FINISH

- Exposed surfaces to be free of scratches and other serious blemishes. Anodic finishes are electrolytically deposited Class II finish. Manufactures standard finish:

  - Clear Anodize – 		ASTM-B-137-95

AAMA 608.1

  - Dark Bronze Anodize - 	ASTM-B-137-95

AAMA 608.1

Note to specifier: Delete finish not required.

- Painted Finishes:

  - 70% Kynar-Base	AA-M10-	AAMA 2605

Duranar or Fluropon	C41-RX	ASCA 96

  - Tiger Drylac RAL #

# Note to specifier: Enter RAL #

Note to specifier: Delete paint finish if not required.

PART 3 – EXECUTION

  - EXAMINATION

- With Installer examine substrates, areas, and conditions for compliance with the requirements for installation tolerances and other conditions affecting functionality and performance of security entrances.

  - Examine rough-in opening for built-in and embedded anchors to verify actual locations of connections before installation.

  - Prepare a written report, endorsed by the installer, listing conditions detrimental to the functionality and performance of security doors.

  - Examine and inspect built-in and cast-in anchors installations to verify that they comply with requirements. Prepare inspection reports. If removal and replacement is required by inspection, re-inspect after repairs are made and document in inspection report.

  - Clean surfaces thoroughly prior to installation.

  - INSTALLATION

- Door assemblies shall be installed in their correct locations, set level, square and plumb in alignment with other work and substrates, in accordance with manufactures instructions, approved shop drawings and accepted industry standards. All joints between door framing and rough opening shall be sealed using sealant to ensure a weather tight installation.

- Installer shall take special care to ensure that impact side of glass faces the threat axis.

- FIELD QUALITY CONTROL

- Inspect installed items to ensure compliance with requirements.

- Protect installed items until completion of project.

END OF SECTION 083453